##################################################################################################

## 

##  Xilinx, Inc. 2010            www.xilinx.com 

##  Thu Feb 9 12:59:49 2017

##  Generated by MIG Version 4.0

##  

##################################################################################################

##  File name :       mig_7series_0.xdc

##  Details :     Constraints file

##                    FPGA Family:       ARTIX7

##                    FPGA Part:         XC7A35T-FTG256

##                    Speedgrade:        -1

##                    Design Entry:      VERILOG

##                    Frequency:         400 MHz

##                    Time Period:       2500 ps

##################################################################################################

##################################################################################################

## Controller 0

## Memory Device: DDR3_SDRAM->Components->MT41K128M16XX-15E

## Data Width: 16

## Time Period: 2500

## Data Mask: 1

##################################################################################################

#create_clock -period 5 [get_ports sys_clk_i]

############## NET - IOSTANDARD ##################

# PadFunction: IO_L13P_T2_MRCC_15 

set_property SLEW FAST [get_ports {ddr3_dq[0]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[0]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[0]}]

set_property PACKAGE_PIN E12 [get_ports {ddr3_dq[0]}]

# PadFunction: IO_L13N_T2_MRCC_15 

set_property SLEW FAST [get_ports {ddr3_dq[1]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[1]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[1]}]

set_property PACKAGE_PIN E13 [get_ports {ddr3_dq[1]}]

# PadFunction: IO_L14P_T2_SRCC_15 

set_property SLEW FAST [get_ports {ddr3_dq[2]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[2]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[2]}]

set_property PACKAGE_PIN E11 [get_ports {ddr3_dq[2]}]

# PadFunction: IO_L14N_T2_SRCC_15 

set_property SLEW FAST [get_ports {ddr3_dq[3]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[3]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[3]}]

set_property PACKAGE_PIN D11 [get_ports {ddr3_dq[3]}]

# PadFunction: IO_L16N_T2_A27_15 

set_property SLEW FAST [get_ports {ddr3_dq[4]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[4]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[4]}]

set_property PACKAGE_PIN F13 [get_ports {ddr3_dq[4]}]

# PadFunction: IO_L17P_T2_A26_15 

set_property SLEW FAST [get_ports {ddr3_dq[5]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[5]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[5]}]

set_property PACKAGE_PIN E16 [get_ports {ddr3_dq[5]}]

# PadFunction: IO_L17N_T2_A25_15 

set_property SLEW FAST [get_ports {ddr3_dq[6]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[6]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[6]}]

set_property PACKAGE_PIN D16 [get_ports {ddr3_dq[6]}]

# PadFunction: IO_L18P_T2_A24_15 

set_property SLEW FAST [get_ports {ddr3_dq[7]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[7]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[7]}]

set_property PACKAGE_PIN F15 [get_ports {ddr3_dq[7]}]

# PadFunction: IO_L19N_T3_A21_VREF_15 

set_property SLEW FAST [get_ports {ddr3_dq[8]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[8]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[8]}]

set_property PACKAGE_PIN G12 [get_ports {ddr3_dq[8]}]

# PadFunction: IO_L20P_T3_A20_15 

set_property SLEW FAST [get_ports {ddr3_dq[9]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[9]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[9]}]

set_property PACKAGE_PIN H12 [get_ports {ddr3_dq[9]}]

# PadFunction: IO_L20N_T3_A19_15 

set_property SLEW FAST [get_ports {ddr3_dq[10]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[10]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[10]}]

set_property PACKAGE_PIN H13 [get_ports {ddr3_dq[10]}]

# PadFunction: IO_L22P_T3_A17_15 

set_property SLEW FAST [get_ports {ddr3_dq[11]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[11]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[11]}]

set_property PACKAGE_PIN H16 [get_ports {ddr3_dq[11]}]

# PadFunction: IO_L22N_T3_A16_15 

set_property SLEW FAST [get_ports {ddr3_dq[12]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[12]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[12]}]

set_property PACKAGE_PIN G16 [get_ports {ddr3_dq[12]}]

# PadFunction: IO_L23P_T3_FOE_B_15 

set_property SLEW FAST [get_ports {ddr3_dq[13]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[13]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[13]}]

set_property PACKAGE_PIN J15 [get_ports {ddr3_dq[13]}]

# PadFunction: IO_L23N_T3_FWE_B_15 

set_property SLEW FAST [get_ports {ddr3_dq[14]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[14]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[14]}]

set_property PACKAGE_PIN J16 [get_ports {ddr3_dq[14]}]

# PadFunction: IO_L24P_T3_RS1_15 

set_property SLEW FAST [get_ports {ddr3_dq[15]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dq[15]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dq[15]}]

set_property PACKAGE_PIN H14 [get_ports {ddr3_dq[15]}]

# PadFunction: IO_L1P_T0_AD0P_15 

set_property SLEW FAST [get_ports {ddr3_addr[13]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[13]}]

set_property PACKAGE_PIN C8 [get_ports {ddr3_addr[13]}]

# PadFunction: IO_L1N_T0_AD0N_15 

set_property SLEW FAST [get_ports {ddr3_addr[12]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[12]}]

set_property PACKAGE_PIN C9 [get_ports {ddr3_addr[12]}]

# PadFunction: IO_L2P_T0_AD8P_15 

set_property SLEW FAST [get_ports {ddr3_addr[11]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[11]}]

set_property PACKAGE_PIN A8 [get_ports {ddr3_addr[11]}]

# PadFunction: IO_L2N_T0_AD8N_15 

set_property SLEW FAST [get_ports {ddr3_addr[10]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[10]}]

set_property PACKAGE_PIN A9 [get_ports {ddr3_addr[10]}]

# PadFunction: IO_L4P_T0_15 

set_property SLEW FAST [get_ports {ddr3_addr[9]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[9]}]

set_property PACKAGE_PIN B10 [get_ports {ddr3_addr[9]}]

# PadFunction: IO_L4N_T0_15 

set_property SLEW FAST [get_ports {ddr3_addr[8]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[8]}]

set_property PACKAGE_PIN B11 [get_ports {ddr3_addr[8]}]

# PadFunction: IO_L5P_T0_AD9P_15 

set_property SLEW FAST [get_ports {ddr3_addr[7]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[7]}]

set_property PACKAGE_PIN B12 [get_ports {ddr3_addr[7]}]

# PadFunction: IO_L5N_T0_AD9N_15 

set_property SLEW FAST [get_ports {ddr3_addr[6]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[6]}]

set_property PACKAGE_PIN A12 [get_ports {ddr3_addr[6]}]

# PadFunction: IO_L6P_T0_15 

set_property SLEW FAST [get_ports {ddr3_addr[5]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[5]}]

set_property PACKAGE_PIN D8 [get_ports {ddr3_addr[5]}]

# PadFunction: IO_L6N_T0_VREF_15 

set_property SLEW FAST [get_ports {ddr3_addr[4]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[4]}]

set_property PACKAGE_PIN D9 [get_ports {ddr3_addr[4]}]

# PadFunction: IO_L7P_T1_AD2P_15 

set_property SLEW FAST [get_ports {ddr3_addr[3]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[3]}]

set_property PACKAGE_PIN A13 [get_ports {ddr3_addr[3]}]

# PadFunction: IO_L7N_T1_AD2N_15 

set_property SLEW FAST [get_ports {ddr3_addr[2]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[2]}]

set_property PACKAGE_PIN A14 [get_ports {ddr3_addr[2]}]

# PadFunction: IO_L8P_T1_AD10P_15 

set_property SLEW FAST [get_ports {ddr3_addr[1]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[1]}]

set_property PACKAGE_PIN C14 [get_ports {ddr3_addr[1]}]

# PadFunction: IO_L8N_T1_AD10N_15 

set_property SLEW FAST [get_ports {ddr3_addr[0]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_addr[0]}]

set_property PACKAGE_PIN B14 [get_ports {ddr3_addr[0]}]

# PadFunction: IO_L9P_T1_DQS_AD3P_15 

set_property SLEW FAST [get_ports {ddr3_ba[2]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_ba[2]}]

set_property PACKAGE_PIN B15 [get_ports {ddr3_ba[2]}]

# PadFunction: IO_L9N_T1_DQS_AD3N_15 

set_property SLEW FAST [get_ports {ddr3_ba[1]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_ba[1]}]

set_property PACKAGE_PIN A15 [get_ports {ddr3_ba[1]}]

# PadFunction: IO_L10P_T1_AD11P_15 

set_property SLEW FAST [get_ports {ddr3_ba[0]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_ba[0]}]

set_property PACKAGE_PIN C16 [get_ports {ddr3_ba[0]}]

# PadFunction: IO_L10N_T1_AD11N_15 

set_property SLEW FAST [get_ports {ddr3_ras_n}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_ras_n}]

set_property PACKAGE_PIN B16 [get_ports {ddr3_ras_n}]

# PadFunction: IO_L11P_T1_SRCC_15 

set_property SLEW FAST [get_ports {ddr3_cas_n}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_cas_n}]

set_property PACKAGE_PIN C11 [get_ports {ddr3_cas_n}]

# PadFunction: IO_L11N_T1_SRCC_15 

set_property SLEW FAST [get_ports {ddr3_we_n}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_we_n}]

set_property PACKAGE_PIN C12 [get_ports {ddr3_we_n}]

# PadFunction: IO_L18N_T2_A23_15 

set_property SLEW FAST [get_ports {ddr3_reset_n}]

set_property IOSTANDARD LVCMOS15 [get_ports {ddr3_reset_n}]

set_property PACKAGE_PIN E15 [get_ports {ddr3_reset_n}]

# PadFunction: IO_L12P_T1_MRCC_15 

set_property SLEW FAST [get_ports {ddr3_cke[0]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_cke[0]}]

set_property PACKAGE_PIN D13 [get_ports {ddr3_cke[0]}]

# PadFunction: IO_L12N_T1_MRCC_15 

set_property SLEW FAST [get_ports {ddr3_odt[0]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_odt[0]}]

set_property PACKAGE_PIN C13 [get_ports {ddr3_odt[0]}]

# PadFunction: IO_L16P_T2_A28_15 

set_property SLEW FAST [get_ports {ddr3_dm[0]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dm[0]}]

set_property PACKAGE_PIN F12 [get_ports {ddr3_dm[0]}]

# PadFunction: IO_L19P_T3_A22_15 

set_property SLEW FAST [get_ports {ddr3_dm[1]}]

set_property IOSTANDARD SSTL15 [get_ports {ddr3_dm[1]}]

set_property PACKAGE_PIN H11 [get_ports {ddr3_dm[1]}]

# PadFunction: IO_L15P_T2_DQS_15 

set_property SLEW FAST [get_ports {ddr3_dqs_p[0]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dqs_p[0]}]

set_property IOSTANDARD DIFF_SSTL15 [get_ports {ddr3_dqs_p[0]}]

set_property PACKAGE_PIN D14 [get_ports {ddr3_dqs_p[0]}]

# PadFunction: IO_L15N_T2_DQS_ADV_B_15 

set_property SLEW FAST [get_ports {ddr3_dqs_n[0]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dqs_n[0]}]

set_property IOSTANDARD DIFF_SSTL15 [get_ports {ddr3_dqs_n[0]}]

set_property PACKAGE_PIN D15 [get_ports {ddr3_dqs_n[0]}]

# PadFunction: IO_L21P_T3_DQS_15 

set_property SLEW FAST [get_ports {ddr3_dqs_p[1]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dqs_p[1]}]

set_property IOSTANDARD DIFF_SSTL15 [get_ports {ddr3_dqs_p[1]}]

set_property PACKAGE_PIN G14 [get_ports {ddr3_dqs_p[1]}]

# PadFunction: IO_L21N_T3_DQS_A18_15 

set_property SLEW FAST [get_ports {ddr3_dqs_n[1]}]

set_property IN_TERM UNTUNED_SPLIT_50 [get_ports {ddr3_dqs_n[1]}]

set_property IOSTANDARD DIFF_SSTL15 [get_ports {ddr3_dqs_n[1]}]

set_property PACKAGE_PIN F14 [get_ports {ddr3_dqs_n[1]}]

# PadFunction: IO_L3P_T0_DQS_AD1P_15 

set_property SLEW FAST [get_ports {ddr3_ck_p[0]}]

set_property IOSTANDARD DIFF_SSTL15 [get_ports {ddr3_ck_p[0]}]

set_property PACKAGE_PIN B9 [get_ports {ddr3_ck_p[0]}]

# PadFunction: IO_L3N_T0_DQS_AD1N_15 

set_property SLEW FAST [get_ports {ddr3_ck_n[0]}]

set_property IOSTANDARD DIFF_SSTL15 [get_ports {ddr3_ck_n[0]}]

set_property PACKAGE_PIN A10 [get_ports {ddr3_ck_n[0]}]

set_property INTERNAL_VREF  0.750 [get_iobanks 15]

set_property LOC PHASER_OUT_PHY_X0Y7 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_D.ddr_byte_lane_D/phaser_out}]

set_property LOC PHASER_OUT_PHY_X0Y6 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_C.ddr_byte_lane_C/phaser_out}]

set_property LOC PHASER_OUT_PHY_X0Y5 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_B.ddr_byte_lane_B/phaser_out}]

set_property LOC PHASER_OUT_PHY_X0Y4 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_A.ddr_byte_lane_A/phaser_out}]

## set_property LOC PHASER_IN_PHY_X0Y7 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_D.ddr_byte_lane_D/phaser_in_gen.phaser_in}]

## set_property LOC PHASER_IN_PHY_X0Y6 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_C.ddr_byte_lane_C/phaser_in_gen.phaser_in}]

set_property LOC PHASER_IN_PHY_X0Y5 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_B.ddr_byte_lane_B/phaser_in_gen.phaser_in}]

set_property LOC PHASER_IN_PHY_X0Y4 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_A.ddr_byte_lane_A/phaser_in_gen.phaser_in}]

set_property LOC OUT_FIFO_X0Y7 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_D.ddr_byte_lane_D/out_fifo}]

set_property LOC OUT_FIFO_X0Y6 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_C.ddr_byte_lane_C/out_fifo}]

set_property LOC OUT_FIFO_X0Y5 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_B.ddr_byte_lane_B/out_fifo}]

set_property LOC OUT_FIFO_X0Y4 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_A.ddr_byte_lane_A/out_fifo}]

set_property LOC IN_FIFO_X0Y5 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_B.ddr_byte_lane_B/in_fifo_gen.in_fifo}]

set_property LOC IN_FIFO_X0Y4 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_A.ddr_byte_lane_A/in_fifo_gen.in_fifo}]

set_property LOC PHY_CONTROL_X0Y1 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/phy_control_i}]

set_property LOC PHASER_REF_X0Y1 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/phaser_ref_i}]

set_property LOC OLOGIC_X0Y69 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_B.ddr_byte_lane_B/ddr_byte_group_io/*slave_ts}]

set_property LOC OLOGIC_X0Y57 [get_cells  -hier -filter {NAME =~ */ddr_phy_4lanes_0.u_ddr_phy_4lanes/ddr_byte_lane_A.ddr_byte_lane_A/ddr_byte_group_io/*slave_ts}]

set_property LOC PLLE2_ADV_X0Y1 [get_cells -hier -filter {NAME =~ */u_ddr3_infrastructure/plle2_i}]

set_property LOC MMCME2_ADV_X0Y1 [get_cells -hier -filter {NAME =~ */u_ddr3_infrastructure/gen_mmcm.mmcm_i}]

set_multicycle_path -from [get_cells -hier -filter {NAME =~ */mc0/mc_read_idle_r_reg}] \

                    -to   [get_cells -hier -filter {NAME =~ */input_[?].iserdes_dq_.iserdesdq}] \

                    -setup 6

set_multicycle_path -from [get_cells -hier -filter {NAME =~ */mc0/mc_read_idle_r_reg}] \

                    -to   [get_cells -hier -filter {NAME =~ */input_[?].iserdes_dq_.iserdesdq}] \

                    -hold 5

set_false_path -through [get_pins -filter {NAME =~ */DQSFOUND} -of [get_cells -hier -filter {REF_NAME == PHASER_IN_PHY}]]

set_multicycle_path -through [get_pins -filter {NAME =~ */OSERDESRST} -of [get_cells -hier -filter {REF_NAME == PHASER_OUT_PHY}]] -setup 2 -start

set_multicycle_path -through [get_pins -filter {NAME =~ */OSERDESRST} -of [get_cells -hier -filter {REF_NAME == PHASER_OUT_PHY}]] -hold 1 -start

set_max_delay -datapath_only -from [get_cells -hier -filter {NAME =~ *temp_mon_enabled.u_tempmon/* && IS_SEQUENTIAL}] -to [get_cells -hier -filter {NAME =~ *temp_mon_enabled.u_tempmon/device_temp_sync_r1*}] 20

set_max_delay -from [get_cells -hier *rstdiv0_sync_r1_reg*] -to [get_pins -filter {NAME =~ */RESET} -of [get_cells -hier -filter {REF_NAME == PHY_CONTROL}]] -datapath_only 5

set_false_path -through [get_pins -hier -filter {NAME =~ */u_iodelay_ctrl/sys_rst}]

set_max_delay -datapath_only -from [get_cells -hier -filter {NAME =~ *ddr3_infrastructure/rstdiv0_sync_r1_reg*}] -to [get_cells -hier -filter {NAME =~ *temp_mon_enabled.u_tempmon/xadc_supplied_temperature.rst_r1*}] 20

